SIDDHARTH GROUP OF INSTITUTIONS :: PUTTUR Siddharth Nagar, Narayanavanam Road – 517583 QUESTION BANK (DESCRIPTIVE) Subject with Code : EMTL (19EC0410) Course & Branch: B.Tech – ECE Year &Sem: II-B.Tech & II-Sem Regulation: R19

UNIT –I

ELECTROSTATIC FIELDS

| # 1 | # a | # Define Coulomb’s law and derive the force F that exists between two unlike charges. |  | # [L1][CO1&2] | [6M] |
| --- | --- | --- | --- | --- | --- |
|  | # b | Three Point Charges Q1=1 mc, Q2=2 mc and Q3=-3 mc are respectively located at (0,0,4), (-2,6,1) and (3,-4,-8). Calculate the electric force and electric field on Q1 due to Q2 and Q3. |  | # [L3][CO1&2] | [6M] |
| # 2 | # a | # Find the electric field at a point P located with a distance of r from an infinite sheet with uniform surface charge density of ρs C/m2. |  | # [L1][CO1&2] | [7M] |
|  | # b | # A Point Charge of 20ηc is Located at the Origin. Determine the Magnitude and direction of the electric field intensity at the Point (1,3,-4). |  | # [L3][CO1&2] | [5M] |
| # 3 | # a | # Define Gauss’s Law. Explain briefly about Maxwell’s 1st equation. |  | # [L1][CO1,2&3] | [8M] |
|  | # b | # What are the advantages and applications of Gauss law? |  | # [L1][CO1&2] | [4M] |
| # 4 | # a | # Apply Gauss Law to evaluate the electric flux density at a point P due to the point charge located at the origin. |  | # [L3][CO1,2&3] | [6M] |
|  | # b | # A Point Charge 100 pC is located at (4,1,-3) while the x-axis carries charge 2ηC/m. If the Plane z=3 is also carries charge 5ηC/m2, find E at (1,1,1). |  | # [L3][CO1&2] | [6M] |
| # 5 | # a | # Evaluate the two Maxwell’s equations for electrostatic fields and state them. |  | # [L5][CO1,2&3] | [9M] |
|  | # b | # List Maxwell equations for electrostatic fields in integral form. |  | # [L1][CO1,2&3] | [3M] |
| # 6 | # a | # Classify Maxwell equations for electrostatic fields in both differential and integral form. |  | # [L4][CO1,2&3] | [6M] |
|  | # b | # Two point charges, QA = +8 μC and QB = -5 μC, are separated by a distance r = 10 cm. What is the magnitude of the electric force between them? |  | # [L3][CO1&2] | [6M] |
| # 7 | # a | Define the Electric Flux Density. Determine the Electric flux density at a point P due to infinite line of uniform Charge density ρL C/m. |  | # [L1][CO1&2] | [8M] |
|  | # b | # Point Charges Q1=4μc ,Q2=-5μc and Q3=2 μc are located at (0,0,1).(-6,8,0) and (0,4,-3) respectively find D at the origin. |  | # [L3][CO1&2] | [4M] |
| # 8 | # a | # Define Eclectic Potential. Find the electric potential for a point charge is located at origin. |  | # [L1][CO1&2] | [8M] |
|  | # b | # Determine the Relationship between E and V. |  | # [L5][CO1&2] | [4M] |
| # 9 | # Explain the following with expression. - Coloumb’s law. b) Electric field intensity. c) Gauss law. |  |  | # [L2][CO1,2&3] | [12M] |
| # 10 | # a |  | Deduce the electric field at a distance r due to an infinitely long straight line of charge with a uniform charge density of 𝝆L 𝐶⁄𝑚. | # [L4] [CO1&2] | [8M] |
|  | # b |  | # A charge of 5×10-8 C is distributed uniformly on the surface of a sphere of radius 1 cm. Calculate the electric flux density. | # [L3] [CO1&2] | [4M] |

UNIT –II

MAGNETOSTATIC FIELDS

| # 1 | # a | # Explain Biot-Savart’s Law. |  | # [L2][CO1&2] | [6M] |
| --- | --- | --- | --- | --- | --- |
|  | # b | A Positive Y-axis (Semi Infinite Line with respect to the Origin) Carries a Filamentary Current of 2 A in the -ay Direction. Assume it is part of a large circuit. Find H at (i) A(2,3,0). (ii) B(3,12,-4). |  | # [L3][CO1&2] | [6M] |
| # 2 | # a | # Explain Ampere’s Circuit Law. |  | # [L2][CO1,2&3] | [6M] |
|  | # b | # Determine the Magnetic Field Intensity due to a infinite sheet current. |  | # [L5][CO1&2] | [6M] |
| # 3 | # a | # Determine Maxwell’s Equations for Magnetostatic Field. |  | # [L5][CO1,2&3] | [6M] |
|  | # b | # Determine the Magnetic Flux Density due to Infinite Sheet of Current. |  | # [L5][CO1&2] | [6M] |
| # 4 | # a | # Discuss about Magnetic Vector and Scalar Potentials. |  | # [L6][CO1&2] | [6M] |
|  | # b | # Given Magnetic Vector potential A= -/4 az wb/m, Calculate the total magnetic flux crossing the =π/2,1≤≤2m, 0≤z≤5m. |  | # [L3][CO1&2] | [6M] |
| # 5 | # a | # Explain about magnetic scalar and vector potential for Magneto-statics. |  | # [L2][CO1&2] | [6M] |
|  | # b | An infinitely filamentary wire carries a current of 2A in the +z direction. Calculate B at (-3,4,7). |  | # [L3][CO1&2] | [6M] |
| # 6 | # a | Determine the Magnetic Field Density due to Infinite line Current by applying Ampere’s Circuit law. |  | # [L5][CO1&2] | [7M] |
|  | # b | # List differential and integral form of Maxwell’s equation for static EM filed. |  | # [L1][CO2&3] | [5M] |
| # 7 | # a | Find the Magnetic field Intensity Due to a Straight current carrying filamentary conductor of finite length. |  | # [L1][CO1&2] | [6M] |
|  | # b | # Find H at (-3,4,0) due to the Current Filament Shown in the Figure. |  | # [L1][CO1,2&3] | [6M] |
| # 8 |  | # Find H for a straight current carrying conductor using Biot Savart’s law and Ampere’s Circuit law. |  | # [L1][CO1&2] | [12M] |
| # 9 | Explain any two applications of Ampere’s Circuit law. |  |  | # [L2][CO1,2&3] | [12M] |
| # 10 | a |  | A Current Distribution gives rise to the vector potential A= X2Yax+Y2Xay+XYZaz web/m. Calculate B. | [L3] [CO1&2] | [6M] |
|  | b |  | Explain about Non-Existence of Magnetic Mono pole. | [L2] [CO2] | [6M] |

UNIT –III

MAXWELL’S EQUATIONS (TIME VARYING FIELDS)

| # 1 | # a | # Explain Faraday’s laws in Electromagnetic induction. | # [L2][CO1&2] | [7M] |
| --- | --- | --- | --- | --- |
|  | # b | Prove that the Displacement Current Density JD=∂D∂t. | # [L5][CO1,2&3] | [5M] |
| # 2 | # a | # Determine the Transformer EMF for the time varying fields. | # [L5][CO1,2&3] | [8M] |
|  | # b | # Define Faraday’s law? | # [L1][CO1,2&3] | [4M] |
| # 3 | # a | Explain Faraday’s law of electromagnetic induction and derive the Expression for Induced EMF. | # [L2][CO1,2&3] | [6M] |
|  | # b | # Explain the motional EMF and derive the expression for the maxwell equation. | # [L2][CO1,2&3] | [6M] |
| # 4 | # a | # Determine the Expressions for inconsistency of Ampere’s law. | # [L5][CO1,2&3] | [9M] |
|  | # b | # Why ampere’s Law is In-consistent. | # [L1][CO2&3] | [3M] |
| # 5 | # a | # Discuss Maxwell’s equation in both differential and integral in final form | # [L6][CO1,2&3] | [7M] |
|  | # b | # An antenna radiates in free space and H= 50 cos(1000t-5y)ax A/m. Calculate ɷ , β and E . | # [L3][CO2&3] | [5M] |
| # 6 | # a | # In free space, E= 20 cos(ɷt-50x)ay V/m. Calculate Jd, H. | # [L3][CO2&3] | [7M] |
|  | # b | # Translate the Maxwell’s equations into word statement. | # [L2][CO1,2&3] | [5M] |
| # 7 | # a | Prove that one of the Maxwell’s equation is E =-dB/dt | # [L5][CO1,2&3] | [7M] |
|  | # b | # In free space, H= 10 sin(ɷt-100x)ay A/m. Calculate Jd, E. | # [L3][CO2&3] | [5M] |
| # 8 | # a | # Prove that one of the Maxwell’s equation is H =Jd+J. | # [L5][CO1,2&3] | [8M] |
|  | # b | # An antenna radiates in free space and E= 80 cos(500t-8z)ax V/m. Calculate ɷ and β. | # [L3][CO2&3] | [4M] |
| # 9 | Explain and determine the EMF for the Followings. - Motional EMF. (ii)Transformer EMF. |  | # [L2][CO2&3] | [12M] |
| # 10 | Explain the following - Faraday’s law ii) Inconsistency of Ampere’s law |  | # [L2] [CO2&3] | [12M] |

UNIT –IV

EM WAVE PROPAGATION

| # 1 |  |  | # Discuss about pointing theorem and poynting vector. | # [L6][CO4&5] | [12M] |
| --- | --- | --- | --- | --- | --- |
| # 2 | # a |  | # Explain and derive the characteristics of wave propagation in free space. | # [L2][CO3,4&5] | [7M] |
|  | # b |  | # Given that E=, Determine the direction of wave propagation, velocity of the wave, wave length. | # [L3][CO4&5] | [5M] |
| # 3 |  |  | # Electric field in free space is given by E= a). Find the direction of wave propagation. b). Calculate and the time it takes to travel a distance of λ. c). Sketch the wave at t=0,T/4 and T/2. | # [L3][CO4&5] | [12M] |
| # 4 | # a |  | # Determine the expression for intrinsic impendence and propagation constant in a good conductor. | # [L5][CO4&5] | [7M] |
|  | # b |  | # In a Nonmagnetic medium E=find,ɳ. | # [L3][CO4&5] | [5M] |
| # 5 | # a |  | # Evaluate the wave characteristics of a uniform plane wave in free space. | # [L5][CO4&5] | [7M] |
|  | # b |  | # In free space (z ≤0), a plane wave with H = 10 cos (t - βz) mA/m is incident normally on a lossless medium (ε=2,µ=8) in region z > 0. Determine the reflected wave and the transmitted wave. | # [L3] [CO4&5] | [5M] |
| # 6 | # a |  | # Evaluate the wave equation in lossy dielectric medium for sinusoidal time variations. | # [L5][CO3,4&5] | [6M] |
|  | # b |  | # In lossless medium H=2 cos (ωt - z) +5sin (ωt - z) .Find , ω, E for the medium. | # [L3][CO4&5] | [6M] |
| # 7 | # a |  | Evaluate the expressions for attenuation constant and phase shift constant of lossy dielectric medium. | # [L5][CO4&5] | [6M] |
|  | # b |  | # A plane wave propagating through medium with has the electric field intensity E = 0.5 sin (t- βz) V/m. Determine wave velocity, wave impedance and magnetic field intensity. | # [L3][CO4&5] | [6M] |
| # 8 |  |  | # Evaluate the expressions for reflection coefficient and transmission coefficient by a normal incident wave for a dielectric medium. | # [L5][CO4&5] | [12M] |
| # 9 |  | # Explain the followings with an expression. # i) Linear polarization ii) Circular polarization iii) Elliptical polarization |  | # [L2][CO4&5] | [12M] |
| # 10 |  | In a medium, E = 14sin (2 X t - 2x) V/m Determine the followings: i) The propagation constant ii) The wavelength # iii) The speed of the wave # iv) Sketch the wave at t=0,T/4 & T/2 |  | # [L3] [CO4&5] | [12M] |

UNIT –V

TRANSMISSION LINES

| # 1 | # a | # Evaluate the equation for voltage and current at any point in a transmission line. | # [L5][CO6] | [7M] |
| --- | --- | --- | --- | --- |
|  | # b | Discuss about Transmission line Parameters. | # [L6][CO6] | [5M] |
| # 2 | # a | # Evaluate the equation for Characteristic Impedance of a Transmission line. | # [L5][CO6] | [6M] |
|  | # b | # A telephone line has the following parameters: R =30 Ω/km, G =0 L = 100mH/km, C = 20µF/m. At 1kHz, calculate the characteristic impedance, propagation constant and velocity of the signal. | # [L3][CO6] | [6M] |
| # 3 | # a | # Explain about S-Circle, r-Circle and x-Circle in smith chart. | # [L2][CO6] | [6M] |
|  | # b | A distortion less line has Z0=60 Ω Attenuation constant = 20 mNp/m and u=0.6c (c is velocity of light) Find the primary parameters of the transmission line(R L C G and λ ) at 100MHz. | # [L3][CO6] | [6M] |
| # 4 | # a | # Evaluate the equation for Input Impedance of the transmission line. | # [L5][CO6] | [6M] |
|  | # b | # A Certain transmission line 2m long operating at ω = 106 rad/s has α=8bd/m, β=1 rad/m, and Z0=60+j40Ω. If the line is connected to a source of 10 ∠00) V, Zg = 40Ω and terminated by a load of 20+j50Ω, determine the input impedance. | # [L3][CO6] | [6M] |
| # 5 | # a | # Relate SWR and reflection coefficient. | # [L2][CO6] | [6M] |
|  | # b | # Explain the applications of transmission lines. | # [L2] [CO6] | [6M] |
| # 6 | # a | # With neat sketch explain about Primary and Secondary constants of transmission line. | # [L2][CO6] | [6M] |
|  | # b | # A low lossless transmission line of 100 Ω characteristics impedance is connected to a load of 200 Ω. Calculate the voltage reflection coefficient and the standing wave ratio. | # [L3][CO6] | [6M] |
| # 7 |  | A 50Ω lossless transmission line is terminated on a load impedance of ZL =(25 + j 50)Ω. Use the smith chart to find. i) Voltage reflection coefficient. ii) VSWR. iii) input impedance of the line, given that the line is 3.3λ long. | # [L3][CO6] | [12M] |
| # 8 | # a | # Explain about the smith chart for finding the SWR and Reflection co-efficient. | # [L2][CO6] | [8M] |
|  | # b | # List out the applications of smith chart? | # [L1][CO6] | [4M] |
| # 9 | A 30 m long lossless transmission line with Z0 = 50Ω operating at 2 MHz is terminated with a load ZL = 60 + j 40Ω. If u = 0.6 C on the line, find the reflection coefficient, the standing wave ratio S and the input impedance. |  | # [L3][CO6] | [12M] |
| # 10 | A lossless transmission line with Z0=50 Ω is 30m long and operates at 3MHz. The line is terminated with a load ZL= 70 +j50Ω, If u=0.6c on the line. Compute reflection coefficient, standing wave ratio and Input impedance, load impedance, SWR and complex reflection coefficient (i) without using smith chart (ii) Using smith chart |  | # [L3][CO6] | [12M] |

# Prepared by:

1. Dr. BASAVARAJ G KUDAMBLE                                                                       Professor/ECE

2. Mr.K BHASKAR                                                                                                 Assistant Professor/ECE

.